Keyes Ai voice control V3.0

1. Overview

- The Keyes Ai voice control V3.0 control board is compatible with the Arduino sensor development platform. There are two USB ports on the control panel, one for the mini USB port and one for the external power supply. The USB to serial port chip of the control board is the CH340 chip, and the driver of CH340 is required to install the driver. The control board comes with a DIP switch for controlling the switch of the external power supply.

- In order to facilitate us to learn to use the Arduino development platform, this control panel is a collection of some sensors and electronic components commonly used in our production, such as active buzzer, infrared receiver, infrared emitter, photoresistor, temperature sensitive resistor, SK6812- P6 RGN lamps, rocker components, etc. At the same time, some additional interfaces are provided on the control panel to facilitate the external connection of other electronic components.

- In order to fix the module to other devices, the control panel also comes with 7 positioning holes.

- Specifications

- Main control chip: ATMEGA328P-AU

- USB to serial port chip: CH340G

- Mini USB interface: upload code interface current is 500mA

- Power supply (recommended): 5V 2A (Micro USB powered)

- Self-contained source buzzer: D7 control

- Comes with 4 SK6812-P6 RGB lights: D11 control

- Comes with infrared receiver: D8 control

- Comes with infrared emitter: D3 control

- Comes with 2 photoresistors: control A6 and A7

- Built-in thermistor: control A3

- Self-supporting rocker components: control A1 and A2

- Comes with two control buttons: KEY1 controls D4, KEY2 controls D5

- Comes with 2 LED lights: D5 control red light and D6 control green light (requires black jumper cap jumper, default is completed)

- Comes with 1 DIP switch and 1 reset button: DIP switch is used to control the power input switch

- Comes with two USB ports: mini USB for uploading programs, micro USB for external power

- Description of each interface and main components

4. Detailed usage

- When using the control panel, we need to install the Arduino IDE, which we can download from the official website of arduino, the link is https://www.arduino.cc/en/Main/OldSoftwareReleases#1.5.x. There are various versions of the IDE in each system. Here we download a Windows system, version 1.5.6 IDE, my favorite version of the IDE, which clicks, on behalf of the direct download software, do not need to install after download, click

- , on behalf of the download is the installation file, we need to install the IDE. We choose to download the installation file, click to download

4.1 Installing the Arduino IDE

- Double click arduino-1.5.6-r2-windows to start.

Select “I Agree”to accept license agreement.

Select components to install and click “Next”.

Click “Browse” and select another folder. Click “Install” to start the installation.

Finally, wait for a few minutes to finish.

4.2 Installing the driver file

- The USB to serial port chip of this control board experiment is CH340G. We need to install the driver of this chip. The driver is usb_ch341_3.1.2009.06. We put the driver on the D disk and start to install the driver. In different systems, the installation method is similar, here we start to install the driver on the win7 system.

- When you connect Keyes Ai voice control V3.0 to your computer the first time, right click “Computer” —>“Properties”—> “Device manager”, you can see “USB2.0-Serial”.

- Click “USB2.0-Serial”, select “Update Driver software”.

- In this page, click “Browse my computer for driver software”.

- Find the “drivers” file.

- Installation completed; click “Close”.

- After driver is installed, go to “Device manager” again. right click “Computer” —>  “Properties”—> “Device manager”, you can see CH340 device as below figure shows, also the correct Com port.

4.3 Arduino IDE Settings and Toolbar Introduction

Open Arduino IDE

To avoid any errors when uploading the program to the board, you must select the correct Arduino board name that matches the board that is connected to your computer. Go to Tools→Board and select your board.

Then select the correct COM port (you can see the corresponding COM port after the driver is successfully installed).

Before our program is uploaded to the board, we must demonstrate the functionality of each symbol that appears in the Arduino IDE toolbar.

A - Used to check for any compilation errors.

B - Used to upload the program to the Arduino board.

C - a shortcut to create a new sketch.

D - Used to open one of the sample sketches directly.

E - to save the sketch.

F - Serial monitor for receiving serial data from the board and sending the serial data to the board.

4.4 Test Control Board

Copy and paste the following program into the Arduino IDE.

//RGB

#include <Adafruit_NeoPixel.h>

#define PIN 11               //RGB control pin

Adafruit_NeoPixel strip = Adafruit_NeoPixel(4, PIN, NEO_GRB + NEO_KHZ800);

Uint8_t brightness = 100; //LED brightness

//RGB ends

//Infrared receiving

#include <IRremote.h>

Int RECV_PIN = 8; //IR receiving pin D8

IRrecv irrecv(RECV_PIN);

Decode_results results;

//Infrared emission

IRsend irsend; //Infrared launch uses this library <IRremote.h> whose pin defaults to D3

//photosensitive

Int sensorPin1 =A6; //Photosensor pin A6

Int sensorPin2 =A7; //Photosensor pin A7

Int CDS1 = 0; // used to store the measured value of the photosensor

Int CDS2 = 0;

//buzzer

Int buzzPin = 7; //Define the connection pin of the buzzer to digital port D7

//Red and green led lights

Const int led_red = 5;

Const int led_green = 6;

//two buttons

Const int key1 = 4;

Const int key2 = 2;

//remote sensing

Int JoyStick_X = A1; //x

Int JoyStick_Y = A2; //y

Char count = 0; //calculate button presses

Char flag = 0; // as a condition for calculating the number of button presses

Char display_flag = 0;

Void setup()

{

pinMode (led_red, OUTPUT); / / set the pin connected to the LED output state

pinMode(led_green,OUTPUT);

digitalWrite(led_red, LOW); //Initialize the led light to a low level, that is, let the led light not light

digitalWrite(led_green,LOW);

pinMode (key1, INPUT); / / set the button connected to the input state

pinMode(key2, INPUT);

//RGB

Strip.begin();

strip.show(); // Initialize all pixels to 'off'

//RGB ends

//remote sensing

pinMode (JoyStick_X, INPUT); / / set the remote sensing connection pin as the input state

pinMode(JoyStick_Y, INPUT);

//The end of remote sensing

//Infrared receiving

irrecv.enableIRIn(); // Start the receiver pinMode(IR_S, OUTPUT); //Set the connection pin of the infrared emission to the output state

//photosensitive

Int sensorPin1 =A6; //Define the control pins of two photosensors as A6 and A7

Int sensorPin2 =A7;

Int CDS1 = 0; //Define two variables, initialized to 0, to assign the values ​​measured by the photosensor to these two variables

Int CDS2 = 0;

//buzzer

pinMode(buzzPin, OUTPUT); //Set the connection pin of the buzzer to the output state

Serial.begin(9600); // Set the baud rate of the serial port to 9600.

}

Void loop()

{

If(digitalRead(key1) == 0) //Check if button 1 is pressed (button 1 is pressed, button1 is equal to 0)

{

Delay(10); //delay 10ms

If(digitalRead(key1) == 0) //Recount if button 1 is pressed again, if it is determined that button 1 is pressed

{

digitalWrite(led_red,HIGH); //led1 light (red light on the board) lights up

}

}

Else

{digitalWrite(led_red,LOW);}

If(digitalRead(key2)==0) //Check if button 2 is pressed

{

Delay(10); //delay 10ms

If(digitalRead(key2) == 0) //Recount if button 2 is pressed again, if it is determined that button 2 is pressed

{

Display_flag=0;

//digitalWrite(led_red, LOW); //led1 light (red light) goes out

//digitalWrite(led_green,HIGH); //led2 light (green light on the board) is on

Flag=1; //As the judgment condition of the next judgment statement, let it execute

}

}

If(flag == 1) //Check if flag is equal to 1

{

While(digitalRead(key2) == 0); //Check if button 2 is released. If it is not released, do not execute the following statement.

Flag = 0; //set flag to 0, and jump out of this statement (if(flag == 1) ) to make it easier to determine if button 2 is pressed.

Count++; //count itself plus 1, calculate the number of times button 2 (button) was pressed

If(count >= 7) //limit the value of count if count is greater than or equal to 8

{

Count = 1; //Reassign count to 0

}

}

Switch (count) / / switch function, used to determine the number of times button 2 is pressed, the function of the corresponding number of times

{

Case 1:

If(display_flag==0)

{Serial.println("IR_test");delay(500);}

Ir_test(); //Execute this subfunction (infrared reception on the board)

Display_flag=1;

Break;

Case 2: //The corresponding button 2 is pressed once, and the following statement is executed.

If(display_flag==0)

{Serial.println("RGB_test");delay(500);} //Print out 1 in the serial monitor

RGB(); //Execute this subfunction (4 RGB lights on the board)

Display_flag=1;

Break; //The value of count is not equal to 1, then exit this subfunction

Case 3: // corresponding button 2 is pressed 2 times

colorWipe(strip.Color(0, 0, 0), 30); //RGB OFF

If(display_flag==0)

{Serial.println("JY_test");delay(500);} //Print out 2 in the serial monitor

Yaogan(); //Execute this subfunction (remote sensing module on the control panel)

Display_flag=1;

Break; //The value of count is not equal to 2, then exit this subfunction

Case 4:

If(display_flag==0)

{Serial.println("CDS_test");delay(500);}

digitalWrite(led_green,LOW);

Guangmin(); //Execute this subfunction (two photosensors on the board)

Display_flag=1;

Break;

Case 5:

If(display_flag==0)

{Serial.println("NTC_test");delay(500); }

Temperature(); //Execute this subfunction (temperature sensor on the board)

Display_flag=1;

Break;

Case 6:

If(display_flag==0)

{Serial.println("BUZZER_test");delay(500); }

Buzzer(); //Execute this subfunction (buzzer on the board)

Display_flag=1;

Break;

Default: / / other does not execute any statement

Break;

}

}

//4 RGB

Void RGB()

{

colorWipe(strip.Color(50, 0, 0), 30);

Delay(300);

colorWipe(strip.Color(0, 50, 0), 30);

Delay(300);

colorWipe(strip.Color(0, 0, 50), 30);

Delay(300);

}

//RGB ends

//remote sensing

Void yaogan()

{

Int x, y; / / define two variables for x and y

x=analogRead(JoyStick_X); // Assign the value of the remote sensing to the X-axis direction to x

y=analogRead(JoyStick_Y); // Assign the value of the remote sensing read to the Y-axis direction to y

Serial.print("X = "); //Print out on the serial monitor: X =

Serial.println(x , DEC); // Print out the decimal value of the value x of the X axis read by the remote sensor in the serial monitor.

Serial.print("Y = "); //Print out in the serial monitor: Y =

Serial.println(y, DEC); // Print out the decimal value of the Y value of the Y axis read by the remote sensor in the serial monitor.

Delay(100); //delay 100ms

}

//Infrared receiving

Void ir_test()

{

Char IR_flag=0;

If(irrecv.decode(&results)) //Detect if data is received

{

If ( results.bits > 0 ) // Whether the number of data bits is greater than 0

{

If ( 0x01 == results.value ) / / Is the received data equal to 0x01

{

digitalWrite(led_green,HIGH);

Delay(500);

IR_flag=1;

digitalWrite(led_green,LOW);

}

Irrecv.resume(); // prepare to receive the next value

}

}

If(IR_flag==1)

{

Irsend.sendRC5(0x01, 8);

Delay(200);

IR_flag=0;

}

irrecv.enableIRIn(); //must

Delay(300);

}

//Two photosensors

Void guangmin()

{

CDS1 = analogRead(sensorPin1); //Read the measured value of a photosensor and assign it to CDS1

CDS2 = analogRead(sensorPin2); //Read the measured value of another photosensor and assign it to CDS2

Serial.print("CDS1 = "); //Print out in the serial monitor: CDS1 =

Serial.println(CDS1, DEC); //Print the measured value of the photosensor in the serial monitor for the decimal number of CDS1

Serial.print("CDS2 = "); //Print out in the serial monitor: CDS2 =

Serial.println(CDS2, DEC); //Print the measured value of the photosensor in the serial monitor for the decimal number of CDS2

Delay(50); //delay 50ms

}

//temperature

Void temperature()

{

Int sensorValue = analogRead(A3); //Read the value measured by the temperature sensor and assign it to sensorValue

Serial.println(sensorValue); // print out the value of sensorValue in the serial monitor.

Delay(1);

}

//buzzer

Void buzzer()

{

digitalWrite(buzzPin, HIGH); //Set the active buzzer's control pin high

Delay(1); //delay 1ms (the length of the delay is different, the sound is different)

digitalWrite(buzzPin, LOW); //Reset the buzzer control pin low

Delay(1); //delay 1ms

}

////////////////////////////////////////////////////////////////////////////////

//RGB LED Fill the dots one after the other with a color

void colorWipe(uint32_t c, uint8_t wait)

{

for(uint16_t i=0; i<strip.numPixels(); i++) {

strip.setPixelColor(i, c);

strip.show();

delay(wait);

}

}

//Theatre-style crawling lights.

void theaterChase(uint32_t c, uint8_t wait) {

for (int j=0; j<10; j++) {  //do 10 cycles of chasing

for (int q=0; q < 3; q++) {

for (int i=0; i < strip.numPixels(); i=i+3) {

strip.setPixelColor(i+q, c);  //turn every third pixel on

}

strip.show();

delay(wait);

for (int i=0; i < strip.numPixels(); i=i+3) {

strip.setPixelColor(i+q, 0);  //turn every third pixel off

}

}

}

}

Before opening the arduino IDE to upload the code, you need to add the Adafruit_NeoPixel.h library file, first find the location of the arduino IDE file, enter the libraries folder, and put the folder with the Adafruit_NeoPixel.h library file into the libraries folder, as shown below.

According to the previous method setting board and COM port, the corresponding lower panel shows the corresponding board and COM port, click the icon  to start compiling the program, check the error, check the error, as shown below.

click starts uploading the program and the upload is successful, as shown below.

After the program is successfully uploaded, the test function is started, and the functions are as follows.

- Press KEY1 on the control panel, the red LED on the control panel will light up, otherwise it will go out.

- ClickOpen the serial port monitor, set the baud rate to 9600, press 1 on the control board KEY2, and start testing the infrared receiving function, as shown below. When the infrared remote control alignment board receives any button from the infrared receiving head, the infrared receiving head receives the information, and the green patch LED near the infrared receiving head lights up.

- Press KEY2 on the control panel again to start testing the control panel for 4 RGB lights. The 4 RGB lights illuminate various colors, and the serial monitor displays as follows.

- Press KEY2 on the control panel again to start testing the joystick components of the control board. The serial monitor displays the analog value corresponding to the X-axis of the joystick, as shown below.

- 5. Press KEY2 on the control board again to start testing the two photoresistors on the control board. The serial monitor displays the analog values of the two photoresistors. The lower the light intensity, the larger the data, as shown below.

- 6. Press KEY2 on the control board again to start testing the temperature sensitive resistor of the control board. The serial monitor displays the analog value of the corresponding temperature sensitive resistor, as shown below.

- Press KEY2 on the control panel again to start testing the active buzzer on the control panel. The buzzer will sound, press KEY2 on the control panel again, and start the infrared receiving function of the test panel, so that we can press continuously. On the lower control board KEY2, select the test function, as shown below.

Schematic