TJ-56-646 function

1. Parameters

Voltage: DC5V

Sec/div: 500ms, 200ms, 100ms, 50ms, 20ms, 10ms, 5ms, 2ms, 1ms, 500us, 200us, 100us

Voltage range: 0-30V (LSB needs to be adjusted)

Sampling level: 10kHz @100us/div

Waveform generator: PWM (PWM generator) and SIN (sine wave generator), adjust RP1 and RP2 to change the amplitude respectively.

2. Main interface description

Trigger slope

网址：http://www.56dz.com

电话：023-58556299

QQ群：549341586

网址：http://www.56dz.com

电话：023-58556299

QQ群：549341586

Trigger Mode

Trigger level

Longitudinal displacement

“Run”: Sampling is running.

“Stop”: Sampling is stopped.

Amplitude

setting

Time per frame

Waveform frequency

Frequency per grid: 500ms, 200ms, 100ms, 50ms, 20ms, 10 ms, 5ms, 2ms, 1ms, 500us, 200us, 100us

Trigger level: trigger voltage level.

Trigger slope: ↑- rising edge, ↓- falling edge trigger

Trigger mode: A- automatic mode, N- normal mode, S- single mode.

"Run": sampling operation.

"Stop": sampling stop.

"Fail": in automatic trigger mode, the trigger level of the waveform is exceeded.

"Auto": automatic voltage range

- Parameter setting interface

PMode (drawing mode): Display waveforms as vectors (Vect) or dots (Dots).

LSB: Input sampling end voltage division coefficient, 100 times voltage division coefficient. By adjusting the voltage division coefficient, the signal of different voltage ranges of 0~30V can be measured.

BRT (brightness): Adjust OLED brightness (25 levels).

Quit: Save and exit.

4. Operation

All operations are completed by the EC11 encoder. Inputs include single click, rotation, and long press rotation.

Single point encoder: Run/stop sampling.

Rotary encoder: Adjust parameters.

Rotate encoder and press at the same time: Switch between options.

Trigger level: For repetitive signals, the trigger level can make it display stably. For single-shot signals, the trigger level

can be captured.

Trigger slope: The trigger slope determines whether the trigger point is on the rising or falling edge of the signal.

Trigger mode:

Automatic mode: Continuous scanning. Click the encoder to stop or run sampling. If triggered, the waveform will be displayed on the display, and the trigger position will be placed in the center of the chart. Otherwise, the waveform will scroll irregularly and display "Fail".

Normal mode: After pre-sampling is completed, the signal can be input. If triggered, the waveform is displayed on the display and waits for a new trigger. If there is no new trigger, the waveform will be retained.

Single mode: After pre-sampling is completed, the signal can be input. If triggered, the waveform is displayed and sampling stops. The user needs to single-point the encoder to start the next sampling.

In normal mode and single mode, confirm that the trigger level has been adjusted correctly, otherwise the waveform will not be displayed on the display.

Indicator: Usually the indicator light is on to indicate that sampling is running.

Save settings: When exiting the setting interface, all parameters of the setting interface and the main interface will be saved in the EEPROM.