Install drive

First, insert the RTL-SDR software radio receiver,

1. Xp prompts to find new hardware, whether to automatically install the driver, select No

2. Windows 7 tips to discover new hardware, while automatically searching for drivers, point to cancel

Second, Install the Zadig driver

- XP system installation zadig-xp; (Note: if the software does not open, please install Microsoft's net 3.5

- other Windows versions (WIN7, WIN8, WIN10) install zadig_win8 (may need to disable digital signatures)

If the system is WIN10, it may also display "RTL2832UHIDIR", select, point Replace Driver.

- Run zading, click options, select list all devices, select bulk-in, interface (interface0) in the drop-down list, the default winusb on the right side of the green arrow, and click instatll driver.

- Check whether the installation is complete. Right click on my computer on the desktop, click Manage, select Device Manager in the list on the left, as shown in the figure below. The driver has been installed.

- If the following figure shows a yellow exclamation mark on the device name, the driver is not installed. Run zading again, select bulk-in, interface (interface 1) in the drop-down list, and then install a driver.

Third, run SDRSharp.exe.

A: Receive 25MHz-1.7GHz VHF and UHF. (FM FM radio, air call, walkie-talkie, TV sound, etc.)

- Connecting the antenna: Connect the antenna to the 25MHz-1.7GHz interface.

- Set: in the upper left corner of the drop-down list, select RTL-SDR / USB, point the right of the configure, check tuner agc. Click Close to exit setting. Click Play (start) in the upper left corner. Use the mouse to drag the red pointer at the top right to adjust the frequency.

- FM radio: 87-108MHz  Select WFM

- Civil walkie-talkie: 136MHz-174MHz,  NFM

400MHz-470MHz,  NFM

- Amateur VHF radio: 50-54MHz, 144-148MHz,  NFM

- Airport Tower Call: 118-135.975MHz,  AM

- Listen to TV sound: 56.25 MHz-957.75MHz,  FM

B: Receive 100KHz-30MHz, including long wave, medium wave, shortwave (broadcast AM, single sideband SSB, equal amplitude CW, etc.):

- Connecting the antenna: Install the outdoor shortwave antenna at 100KHz-30MHz interface. (The donated antenna can not be used for shortwave reception)

- Set: Stop state, click Configure, select Direct sampling (Q branch) under Sampling Mode Check RTL AGC and click Close, then click Play. Transferred to the required frequency, select the corresponding modulation:

Long wave: 300KHz or less

Medium wave radio: 526.5kHz ~ 1606.5kHz, select AM

Short wave radio: 3MHz-30MHz, select AM

Short-wave SSB communication: 10MHz the following commonly used under the sideband LSB; 10MHz or more commonly used on a single sideband USB

Note: when you receive WFM, the software automatically exits, or the software does not open, please install Microsoft's Net Framework 3.5

Antenna can be made with reference to the following picture: