一、 Product Settings

Product Use This product primarily displays time, temperature, and humidity. It can be used in living rooms, bedrooms, classrooms, offices, production workshops, etc. It is not recommended for outdoor use as it may lead to inaccurate display.

二、Product Features

Simple design, dimensions: 11x6x1.8cm, simple and compact design.

Simultaneous display of time, temperature, and humidity on the same screen.

4-inch LCD large screen display.

Accurate display: temperature display ±1.0℃, accuracy: ±1℃.

三、Instructions for Use

Button Functions SET button: Long press this button for about 3 seconds to enter time setting. ADJ button: Adjust the setting object during setting. 'C/℉' button: Switch temperature display units. RESET button: Restore factory settings.

Setting Instructions In normal display mode, the “.” between hours and minutes flashes once per second. Press the SET button for about 3 seconds to enter the setting interface, short press the “ADJ” button to adjust the setting items. Note: The setting order is: Minutes → Hours → 12/24 → Year → Month → Day → Exit. Use the ADJ button to adjust the minutes setting. When setting the time, long press the ADJ button to quickly adjust at approximately 8HZ.

四、Product Parameters

Product Size     Approximately 11x6x1.8cm

Screen          4-inch LCD screen

Temperature Measurement Range        Display range: -10℃~60℃,

Temperature Measurement Accuracy     ±1℃ (0℃~40℃)

Temperature Resolution        0.1℃

Humidity Measurement Range        10%RH~99%RH

Humidity Measurement Accuracy   ±5%RH (40%RH~70%RH), in other ranges max ±7%RH

Humidity Resolution        1%RH

12/24 Hour Display        Yes

Date Reminder            Yes

Battery Type              1.5V AAA battery

Product Weight           Approximately 92 grams

Shell Material            Plastic (ABS)

Product package includes:

main unit, 1.5V AAA battery, product line, manual, top rod