1. Protocol Introduction

The communication protocol is MODBUS-RTU protocol. This product only supports function codes 0x03, 0x06, and 0x10. The communication interface is TTL serial port.

2. Communication protocol introduction

Information transmission is asynchronous. The Modbus-RTU mode uses 11-bit bytes as units.

| Word format (serial data) | 10-bit binary |
| --- | --- |
| Start bit | 1 bit |
| Data bit | 8 bits |
| Parity bit | None |
| Stop bit | 1 bit |

Data frame structure:

| Data frame interval | Address code | Function code | Data area | CRC check |
| --- | --- | --- | --- | --- |
| 3.5 bytes or more | 1 byte | 1 byte | N byte | 2 byte |

Before sending data, the data bus static time is required to be greater than 3.5 (for example: 5ms when the baud rate is 9600). The message transmission must start with a pause interval of at least 3.5 bytes. The entire message frame must be a continuous data transmission stream. If there is a pause time of more than 3.5 bytes before the frame is completed, the receiving device will refresh the incomplete message and assume that the next byte is the address field of a new message. Similarly, if a new message starts after the previous message in less than 3.5 characters, the receiving device will consider it to be a continuation of the previous message.

1.1 Address code:

The address code is the first byte (8 bits) of each communication information frame, from 1 to 255. This byte indicates that the slave with the address set by the user will receive the information sent by the host. Each slave must have a unique address code, and only the slave that meets the address code can respond to the return message. When the slave returns information, the return data starts with its own address code. The address code sent by the host indicates the address of the slave to be sent, and the address code returned by the slave indicates the address of the slave to be sent back. The corresponding address code indicates where the information comes from.

1.2 Function code:

The function code is the second byte transmitted in each communication information frame. The function code that can be defined in the ModBus communication protocol is 1 to 127. As a host request to send, the function code tells the slave what action to perform. As a slave response, the function code returned by the slave is the same as the function code sent from the host, and indicates that the slave has responded to the host and has performed related operations.

This machine only supports 0x03, 0x06, and 0x10 function codes.

| Function code | Definition | Operation (binary) |
| --- | --- | --- |
| 0x03 | Read register data | Read data from one or more registers |
| 0x06 | Write single register | Write a set of binary data to a single register |
| 0x10 | Write multiple registers | Write multiple sets of binary data to multiple registers |

1.3 Data Area

The data area includes what information needs to be returned by the slave or what action needs to be performed. This information can be data (such as: switch input/output, analog input/output, register, etc.), reference address, etc. For example, the host tells the slave to return the value of the register (including the starting address of the register to be read and the length of the register to be read) through function code 03, and the returned data includes the data length and data content of the register.

0x03 Read function host format

| Address code | Function code | Register starting address | Register address number n (1 to 32) | CRC check |
| --- | --- | --- | --- | --- |
| 1 byte | 1 byte | 2 byte | 2 byte | 2 byte |

0x03 Read function slave return format

| Address code | Function code | Return the number of registers n*2 | Register data | CRC check |
| --- | --- | --- | --- | --- |
| 1 byte | 1 byte | 1 byte | 2*n byte | 2 byte |

0x06 Write single register function host format

| Address code | Function code | Register starting address | Register data | CRC check |
| --- | --- | --- | --- | --- |
| 1 byte | 1 byte | 2 byte | 2 byte | 2 byte |

0x06 Write single register function slave return format

| Address code | Function code | Register starting address | Register data | CRC check |
| --- | --- | --- | --- | --- |
| 1 byte | 1 byte | 2 byte | 2 byte | 2 byte |

0x10 Write multiple registers function host format

| Address code | Function code | Register starting address | Number of register addresses n (1~32) | Write byte number 2*n | Register data | CRC check |
| --- | --- | --- | --- | --- | --- | --- |
| 1 byte | 1 byte | 2 byte | 2 byte | 1 byte | 2*N byte | 2 byte |

0x10 Write multiple registers from the master format

| Address code | Function code | Register starting address | Number of register addresses n (1 to 32) | CRC check |
| --- | --- | --- | --- | --- |
| 1 byte | 1 byte | 2 byte | 1 byte | 2 byte |

Factory default baud rate 115200 Device address 1

Protocol register introduction (the data in a single register address is double byte data)

| Name | Description | Number of bytes | Decimal point | Unit | Read/write | Register address (decimal) | Register address (hexadecimal) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| V-SET | Voltage setting | 2 | 2 | V | R/W | 0 | 0x0000 |
| I-SET | Current setting | 2 | 3 | A | R/W | 1 | 0x0001 |
| VOUT | Output voltage display value | 2 | 2 | V | R | 2 | 0x0002 |
| IOUT | Output current display value | 2 | 3 | A | R | 3 | 0x0003 |
| POWER | Output power display value | 2 | 2 | W | R | 4 | 0x0004 |
| UIN | Input voltage display value | 2 | 2 | V | R | 5 | 0x0005 |
| AH-LOW | Output AH low 16 bits | 2 | 0 | maH | R | 6 | 0x0006 |
| AH-HIGH | Output AH high 16 bits | 2 | 0 | maH | R | 7 | 0x0007 |
| WH-LOW | Output WH low 16 bits | 2 | 0 | mwH | R | 8 | 0x0008 |
| WH-HIGH | Output WH high 16 bits | 2 | 0 | mwH | R | 9 | 0x0009 |
| OUT_H | On time - hours | 2 | 0 | H | R | 10 | 0x000A |
| OUT_M | Start time - minutes | 2 | 0 | M | R | 11 | 0x000B |
| OUT_S | On time - seconds | 2 | 0 | S | R | 12 | 0x000C |
| T_IN | Internal temperature value | 2 | 1 | F/C | R | 13 | 0x000D |
| T_EX | External temperature value | 2 | 1 | F/C | R | 14 | 0x000E |
| LOCK | Key lock | 2 | 0 | - | R/W | 15 | 0x000F |
| PROTECT | Protection status | 2 | 0 | - | R/W | 16 | 0x0010 |
| CVCC | Constant voltage and constant current status | 2 | 0 | - | R | 17 | 0x0011 |
| ONOFF | Switch output | 2 | 0 | - | R/W | 18 | 0x0012 |
| F-C | Temperature symbol | 2 | 0 | - | R/W | 19 | 0x0013 |
| B-LED | Backlight brightness level | 2 | 0 | - | R/W | 20 | 0x0014 |
| SLEEP | Screen off time | 2 | 0 | M | R/W | 21 | 0x0015 |
| MODEL | Product model | 2 | 0 | - | R | 22 | 0x0016 |
| VERSION | Firmware version number | 2 | 0 | - | R | 23 | 0x0017 |
| SLAVE-ADD | Slave address | 2 | 0 | - | R/W | 24 | 0x0018 |
| BAUDRATE_L | Baud rate | 2 | 0 | - | R/W | 25 | 0x0019 |
| T-IN-OFFSET | Internal temperature correction | 2 | 1 | F/C | R/W | 26 | 0x001A |
| T-EX-OFFSET | External temperature correction | 2 | 1 | F/C | R/W | 27 | 0x001B |
| BUZZER | Buzzer switch | 2 | 0 | - | R/W | 28 | 0x001C |
| EXTRACT-M | Quickly call up data group | 2 | 0 | - | R/W | 29 | 0x001D |
| DEVICE | Device status | 2 | 0 | - | R/W | 30 | 0x001E |
| MPPT-SW | MPPT switch | 2 | 0 | - | R/W | 31 | 0x001F |
| MPPT-K | MPPT maximum power point coefficient | 2 | 0 | - | R/W | 32 | 0x0020 |
| BatFul | Full power cut-off current | 2 | 0 | - | R/W | 33 | 0x0021 |
| CW-SW | Constant power switch | 2 | 0 | - | R/W | 34 | 0x0022 |
| CW | Constant power value | 2 | 0 | - | R/W | 35 | 0x0023 |
| MASTER | Host type | 2 | 0 | 0 | R/W | 48 | 0x0030 |
| WIFI-CONFIG | WIFI network configuration function | 2 | 0 | 0 | R/W | 49 | 0x0031 |
| WIFI-STATUS | WIFI status | 2 | 0 | 0 | R/W | 50 | 0x0032 |
| IPV4-H | The first two bytes of IP address | 2 | 0 | 0 | R/W | 51 | 0x0033 |
| IPV4-L | The last two bytes of IP address | 2 | 0 | 0 | R/W | 52 | 0x0034 |
| V-SET | Voltage setting | 2 | 2 | V | R/W | 80 | 0x0050 |
| I-SET | Current setting | 2 | 3 | A | R/W | 81 | 0x0051 |
| S-LVP | Low voltage protection value | 2 | 2 | V | R/W | 82 | 0x0052 |
| S-OVP | Over voltage protection value | 2 | 2 | V | R/W | 83 | 0x0053 |
| S-OCP | Over current protection value | 2 | 3 | A | R/W | 84 | 0x0054 |
| S-OPP | Over power protection value | 2 | 1 | W | R/W | 85 | 0x0055 |
| S-OHP_H | Maximum output duration - hours | 2 | 0 | H | R/W | 86 | 0x0056 |
| S-OHP_M | Maximum output duration - minutes | 2 | 0 | M | R/W | 87 | 0x0057 |
| S-OAH_L | Maximum output AH low 16 bits | 2 | 0 | maH | R/W | 88 | 0x0058 |
| S-OAH_H | Maximum output AH high 16 bits | 2 | 0 | maH | R/W | 89 | 0x0059 |
| S-OWH_L | Maximum output WH low 16 bits | 2 | 0 | 10mwH | R/W | 90 | 0x005A |
| S-OWH_H | Maximum output WH high 16 bits | 2 | 0 | 10mwH | R/W | 91 | 0x005B |
| S-OTP | Over temperature protection value | 2 | 0 | F/C | R/W | 92 | 0x005C |
| S-INI | Power-on output switch | 2 | 0 | - | R/W | 93 | 0x005D |
| S-ETP | External over temperature protection | 2 | 0 | - | R/W | 94 | 0x005E |

Note 1: (0019H) Baud rate register meaning 0:9600 1:14400 2:19200 3:38400 4:56000 5:576000 6:115200 (7:2400 8:4800 supported by some devices)

Note 2: This product is designed with 10 storage data groups M0-M9, each group has 14 data in total, including serial numbers 20-2D, of which M0 data group is the default data group called when the product is powered on, M1 and M2 data groups are quick call data groups on the product panel, M3-M9 are ordinary storage arrays, and the starting address of the data group is calculated as follows: 0050H+data group number*0010H, for example, the starting address of the M3 data group is: 0050H+3*0010H=0080H.

Note 3: The read and write values of the key lock function are 0 and 1, 0 is unlocked, and 1 is locked.

Note 4: Protection status register:

0: Normal operation, 1: OVP, 2: OCP, 3: OPP, 4: LVP, 5: OAH, 6: OHP, 7: OTP, 8: OEP, 9: OWH, 10: ICP 11: ETP

| 0: Alarm code | 1: OVP overvoltage protection | 2: OCP overcurrent protection | 3: OPP over-power protection |
| --- | --- | --- | --- |
| 4: LVP input undervoltage protection | 5: OAH maximum output capacity | 6: OHP maximum output time | 7: OTP over-temperature protection |
| 8: OEP no output protection | 9: OWH maximum energy output | 10: ICP maximum input current protection | 11: ETP external temperature protection |

Note 5: The constant voltage and constant current state reading values are 0 and 1, 0 is CV state, 1 is CC state.

Note 6: The switch output function read and write values are 0 and 1, 0 is off state, 1 is on state.

Note 7: The backlight brightness level read and write range is 0-5, 0 is the darkest, 5 is the brightest.

Note 8: The quick call data group function write value is 0-9, and the corresponding data group data will be automatically called out after writing.

Note 9: Description of WiFi related registers.

| Name | Detailed description | Register Address |
| --- | --- | --- |
| MASTER | Host type (0x3B3A: WIFI, others to be determined) | 0030H |
| WIFI-CONFIG | WIFI pairing status (0: invalid 1: touch pairing 2: AP pairing) | 0031H |
| WIFI-STATUS | WIFI status (0: invalid network 1: connected to router 2: successfully connected to server 3: touch pairing 4: AP pairing) | 0032H |
| IPV4-H | The first two bytes of the IP address are 0xC0A8 | 0033H |
| IPV4-L | The last two bytes of the IP address are 0x0108 | 0034H |

IPV4-H: 0xC0A8 IPV4-L: 0x0108

IPV4 = 192.168.1.8

1.4 Error check code (CRC check):

The host or slave can use the check code to determine whether the received information is correct. Due to electronic noise or some other interference, errors may occur during the transmission of information. The error check code (CRC) can check whether the information of the host or slave is incorrect during the communication data transmission process. The wrong data can be discarded (whether sending or receiving), which increases the safety and efficiency of the system. The CRC (redundant cyclic code) of the MODBUS communication protocol contains 2 bytes, that is, a 16-bit binary number.

The CRC code is calculated by the sending device (host) and placed at the end of the sent information frame. The device (slave) receiving the information recalculates the CRC of the received information and compares whether the calculated CRC matches the received one. If the two do not match, it indicates an error. When the CRC check code is sent, the low bit is in front and the high bit is in the back.

CRC code calculation method:

(1) Preset a 16-bit register to hexadecimal FFFF (i.e. all 1); call this register the CRC register;

(2) XOR the first 8-bit binary data (i.e. the first byte of the communication information frame) with the lower 8 bits of the 16-bit CRC register, and put the result in the CRC register;

(3) Shift the content of the CRC register right by one bit (towards the lower bit), fill the highest bit with 0, and check the shifted bit after right shift;

(4) If the shifted bit is 0: repeat step 3 (shift right by one bit again); if the shifted bit is 1: CRC register and polynomial A001 (1010 0000 0000) 0001) for XOR;

(5) Repeat steps 3 and 4 until right shifted 8 times, so that the entire 8-bit data is processed;

(6) Repeat steps 2 to 5 to process the next byte of the communication information frame;

(7) After all bytes of the communication information frame are calculated according to the above steps, the high and low bytes of the 16-bit CRC register are exchanged;

(8) The final content of the CRC register is the CRC code.

III. Communication Examples

Example 1: The host reads the output voltage and output current display values

The message format sent by the host:

| Host sends | Number of byte | Information sent | Notes |
| --- | --- | --- | --- |
| Slave address | 1 | 01 | Sent to slave with address 01 |
| Function code | 1 | 03 | Read register |
| Register starting address | 2 | 0002H | Register starting address |
| Number of register addresses | 2 | 0002H | Total 2 bytes |
| CRC code | 2 | 65CBH | CRC code calculated by the host |

For example, if the current displayed value is 05.00V, 1.500A, the message format returned by the slave response is:

| Slave response | Number of byte | Information sent | Notes |
| --- | --- | --- | --- |
| Slave address | 1 | 01 | From slave 01 |
| Function code | 1 | 03 | Read register |
| Number of bytes to read | 1 | 04 | Total 1 byte |
| Contents of register at address 0002H | 2 | 01F4H | Output voltage display value |
| Contents of register at address 0003H | 2 | 05DCH | Output current display value |
| CRC code | 2 | B8F4H | CRC code calculated by slave |

Example 2: The host wants to set the voltage to 24.00V

The message format sent by the host is:

| Host sends | Number of byte | Information sent | Notes |
| --- | --- | --- | --- |
| Slave address | 1 | 01H | From slave 01 |
| Function code | 1 | 06H | Write a single register |
| Register address | 2 | 0000H | Register address |
| The content of the register with address 0000H | 2 | 0960H | Set output voltage value |
| CRC code | 2 | 8FB2H | CRC code calculated by the host |

The message format returned by the slave after receiving the response is:

| Slave response | Number of byte | Information sent | Notes |
| --- | --- | --- | --- |
| Slave address | 1 | 01H | Send to slave with address 01 |
| Function code | 1 | 06H | Write single register |
| Register address | 2 | 0000H | Register starting address |
| Contents of register with address 0000H | 2 | 0960H | Set output voltage value |
| CRC code | 2 | 8FB2H | CRC code calculated by slave |

Example 3: The host wants to set the voltage to 24.00V and the current to 15.00A.

The message format sent by the host is:

| Host sends | Number of byte | Information sent | Notes |
| --- | --- | --- | --- |
| Slave address | 1 | 01H | From slave 01 |
| Function code | 1 | 10H | Write register |
| Register starting address | 2 | 0000H | Register starting address |
| Number of register addresses | 2 | 0002H | Total 2 bytes |
| Number of bytes written | 1 | 04H | Total 1 byte |
| Contents of register with address 0000H | 2 | 0960H | Set output voltage value |
| Contents of register with address 0001H | 2 | 05DCH | Set output current value |
| CRC code | 2 | F2E4H | Calculate CRC code by host |

The message format returned by the slave after receiving the response is:

| Slave response | Number of byte | Information sent | Notes |
| --- | --- | --- | --- |
| Slave address | 1 | 01H | Send to the slave with address 01 |
| Function code | 1 | 10H | Write register |
| Register starting address | 2 | 0000H | Register starting address |
| Number of register addresses | 2 | 0002H | Total 2 bytes |
| CRC code | 2 | 41C8H | CRC code calculated by the slave |