1 Channel Bluetooth Relay

I. Features:

1 Operating voltage: power supply 1, DC 6-25V; power supply 2,C 5V

2 Operating Current: Standby 11MA, Relay Open 45MA (voltage DC 12V)

3 Bluetooth V2.1 + EDR, Class 2

4 2.4GHz-2.48GHz unlicensed ISM band

5send commands via  Android Bluetooth APP control relay turned on and off (we provide Bluetooth APP, you can also use your own APP to control relays, sending  commands to control),It can also be triggered by an external pulse signal.

6 "Momentary" "Self-locking"  "open" "close" 4 Commands

7 External trigger input, opto-isolated switch, low pulse inputs.

8 size (including housing) 81 * 50 * 32

9 Relay Maximum load:10A/250VAC ,10A/125VAC,10A /30VDC, 10A /28VDC ,10A /12VDC

NO         : Relay normally open contact

COM       : Relay common contact

NC         : Relay normally closed contact

Momentary  : Mobile terminal input "A0", the relay is "open",and     after delay of 1 second the relay is "Close".

Self-locking  : Mobile terminal input "A1", the relay is "Open", input "A1", the relay is "Close", and so on

Open      : NO connection COM, NC disconnect COM

Close      : NO disconnect COM, NC connection COM

External trigger input: low pulse triggering, pulse width greater than 200 milliseconds.

Two control methods, external trigger signal and Command Control,It can be composed of flexible and diverse control.

CommandS (Format: character, case-insensitive)

| CommandS | Features | Description |
| --- | --- | --- |
| A0 | "Momentary" | Mobile terminal input "A0", the relay is "open",and after delay of 1 second the relay is "Close". |
| A1 | " Self-locking " | Mobile terminal input "A1", the relay is "Open", input "A1", the relay is "Close", and so on |
| A2 | " Open " | NO1 connection COM1, NC1 disconnect COM1 |
| A3 | " Close " | NO1 disconnect COM1, NC1 connection COM1 |
| A4 | " Open " | NO1 connection COM1, NC1 disconnect COM1 |

Bluetooth APP operations：

Access Password:1234

Bluetooth Baud rate: 9600BPS

Video (shooting bad, just live with) : http://v.youku.com/v_show/id_XMTI5NzczOTY5Ng==.html

Typical applications:

1 Power supply is DC 6-25V, "LOAD" also DC 6-25V circuit, wiring diagram below.

"LOAD" may be LED lights, fans, motors and other  DC 6-25V equipment

2 Power supply is DC 6-25V, "LOAD" is DC 1-48V or AC 85-265V circuit, wiring diagram below.

"LOAD" may be LED lights, fans, motors and other DC 1-48V or AC 85-265V  equipment

3 Power supply is DC5V, "LOAD" is DC 1-48V or AC 85-265V circuit, wiring diagram below.

"LOAD" may be LED lights, fans, motors and other DC 1-48V or AC 85-265V  equipment

4 Power supply is DC 5V, "LOAD" is DC 5V circuit, wiring diagram below.

"LOAD" may be LED lights, fans, mcu Development Boards and other DC 5V equipment