The accuracy of this signal source and professional comprehensive test are basically the same.

The precision is very high.

It can test the actual receiving sensitivity of the walkie-talkie.

The unit is DBM

General Handset Sensitivity -120DBM to -130DBM

The larger the sensitivity is, the greater the negative value

higher sensitivity.

The signal source cannot input power.

It is absolutely forbidden to press the transmitter key of the walkie-talkie during the test. (Self-configured power supply 8V-12V power supply polarity is positive and negative outside)

testing method

during testing. The frequency of the signal source input into the walkie-talkie is firstly set to -100DB or any value.

The walkie-talkie has an audio output and then reduces the output strength of the signal source. For example, when I arrived at -120DB, I just heard the audio signal of the walkie-talkie but there was noise. If the audio signal can be heard clearly, the -120DB value of this output is the receiving sensitivity of the walkie-talkie.

Specially used for FM walkie-talkie debugging

Frequency 0.5M---470M

Signal strength -70--132dBm

Built-in 800Hz audio modulation

Use 8--12V DC power frequency 7 digits

For example: 439.6000MHZ displays 4396000KHZ

Signal strength -70--132dBm is valid Other values are invalid

Audio modulation is only about 800Hz other values are invalid

F1: Circularly select the setting item "<" to point to the selected item, respectively frequency, signal strength, audio modulation frequency

F2: Backspace key on typo

F3: close, open the signal output

F4: Turn off and turn on audio signal modulation

Numeric keys: directly input the value of the selected item

Up and down keys: step value to modify the value of the selected item

Note: It is absolutely forbidden to press the transmitter key of the walkie-talkie during the test.